FREQUENTLY ASKED QUESTIONS-IMM

Contents

1.	Summary	2

2.	How to connect an OPC UA client to the Wittmann 4.0 router via IMM B8 using port 14970?	2

# Summary

This document summarizes frequently asked questions regarding the connection setup, configuration, and data interface for connecting an OPC UA client to the Wittmann 4.0 router via IMM B8.

# How to connect an OPC UA client to the Wittmann 4.0 router via IMM B8 using port 14970?

- Ensure that the OPC UA client is connected to the Wittmann 4.0 Router before initiating the connection to the IMM B8

- Navigate to UaExpert.

- On the main screen, click the plus button highlighted in the image below to open the Add Server window.

- Double-click the entry under Custom Discovery highlighted below

Note: Use port 14970 in Euromap 77 OPC UA to connect to the IMM.
Click OK to connect.

- Click Yes on the following message box

- Double-click on None - None (uatcp-uasc-uabinary), then click OK to connect.

- Check the Trust Server Certificate checkbox, then click Continue.

- Right-click on Aggregation ServerOpcUa@com-x-router and select Properties.

- A Server Settings pop-up will appear on the screen. Change the default port from 48020 to14970 for the Euromap 77 OPC UA connection.

- Then, click OK to continue.

- Right-click on Aggregation ServerOpcUa@com-x-router, then click Connect.

- You may receive a “URL not found” pop-up. Click OK to continue.

- Check the Trust Server Certificate checkbox, then click Continue.

- Drag and drop the required Node Id into the Data Access View panel

Example- DeviceSet

Note: If any pop-up dialog appears while dragging the Node Id to the Data Access View, click Yes.

- The Nodes will be listed in the Data Access View table, completing the setup for monitoring