# Technical product information

TPI_2016004

Date: 14/03/2016

Subject: Tightening torque Screw connections

Production line: All product lines

Contents

- General screw connection	1

- Screw retention	2

- Stud bolts	3

  - Fastening screws - nozzle head	4

  - Torque settings for the fastening screws of the screw barrel head	5

- Screw coupling SPE 2250-8800	6

## General screw connection

The defined tightening torques are based on a friction coefficient of µtot = 0.125 material metallic bright or galvanised lubricant, graphite, greases, oils.

### Standard Tightening Torque Values for Metric Bolts (Property Classes 8.8, 10.9, and 12.9)

|  |  | Tightening torque Ma in Nm |  |  |
| --- | --- | --- | --- | --- |
|  |  | Property class |  |  |
| Thread | Friction (Standard coefficient thread) µtot | 8.8 | 10.9 | 12.9 |
| M3 | 0.125 | 1.28 | 1.8 | 2.15 |
| M4 | 0.125 | 2.9 | 4.1 | 4.95 |
| M5 | 0.125 | 5.75 | 8.1 | 9.7 |
| M6 | 0.125 | 9.9 | 14 | 16.5 |
| M8 | 0.125 | 24 | 34 | 40 |
| M10 | 0.125 | 48 | 67 | 81 |
| M12 | 0.125 | 83 | 117 | 140 |
| M14 | 0.125 | 132 | 185 | 220 |
| M16 | 0.125 | 200 | 285 | 340 |
| M18 | 0.125 | 275 | 390 | 470 |
| M20 | 0.125 | 390 | 550 | 660 |
| M22 | 0.125 | 530 | 745 | 890 |
| M24 | 0.125 | 675 | 950 | 1140 |
| M27 | 0.125 | 995 | 1400 | 1680 |
| M30 | 0.125 | 1350 | 1900 | 2280 |
| M33 | 0.125 | 1830 | 2580 | 3090 |
| M36 | 0.125 | 2360 | 3310 | 3980 |
| M39 | 0.125 | 3050 | 4290 | 5150 |

## Screw retention

- If not secured by another screw retention Schnorr tooth lock washer, spring washer, etc.

- Loctite 243 must, in principle, be used to secure screws.

Exceptionally, Loctite 638 hydraulic screw connections and Loctite 542 shaft-hub-connections can be used, too.

### Exceptions

- As well as already today, these standard values will, however, not be applicable to special tightening torques nozzle head, nozzle movement cylinders, etc.

- Starting with the Power series, special torques and/or different screw retentions are separately specified in the DOC drawing and the parts list together with the respective screw connection.

- Since the tightening moments of old series HM,TM, etc cannot be seen in the DOC drawing, contact the technical department in cases of doubt.

## Stud bolts

Enclosed is a list of the torques to be used for stud bolts. Values differing from this table are separately given in the parts list and in the DOC drawing.

|  | Tightening torque Ma in Nm |  |
| --- | --- | --- |
| Thread | Shaft screw 8.8 | Stud bolt 8.8 |
| M8 | 24 | 18 |
| M10 | 48 | 36 |
| M12 | 83 | 60 |
| M16 | 200 | 140 |
| M20 | 390 | 275 |
| M24 | 675 | 500 |
| M30 | 1350 | 1000 |
| M36 | 2360 | 1700 |

  - Fastening screws - nozzle head

    - For high-temperature connections, such as on the nozzle head, WEICON ANTI-SEIZE must be applied to the screws.

    - Due to the constriction in the front area, the tightening torque of the nozzle head screws are reduced by approx. 15-20 % to prevent this effect.

    - To avoid any leakage between screw barrel and nozzle head surface pressure between the two components is also lower due to the reduced torques

### The screws must be tightened in 3 steps

- by hand

- 60 %

- 100% and diagonally.

  - These reduced tightening torques must be used as of now for all new machines and constructure changes by the customer service.

  - No action is necessary for existing customer machines since no leakage is expected here.

  - Here is the updated list of the tightening torques for the nozzle head screws:

  - Torque settings for the fastening screws of the screw barrel head

    - These torque settings are valid for socket head cap screws 1 for fastening the nozzle head 2

    - Further information on tensile strengths etc. can be found in the spare parts lists.

Fastening Screws for the Screw Barrel Head

### Standard version

| Thread Nominal Size | Tightening torque Assembly property class 12.G |  |
| --- | --- | --- |
|  | [NM] | [ft lb] |
| M8 | 28 | 21 |
| M12 | 110 | 81 |
| M16 | 250 | 184 |
| M20 | 470 | 347 |
| M24 | 800 | 590 |
| M30 | 1700 | 1254 |

### High temperature heater hands

| Thread Nominal Size | Tightening torque Assembly property class GB |  |
| --- | --- | --- |
|  | [NM] | [ft lb] |
| M8 | 25 | 18 |
| M12 | 90 | 66 |
| M16 | 200 | 148 |
| M20 | 350 | 258 |
| M24 | 650 | 479 |
| M30 | 1280 | 944 |

## Screw coupling SPE 2250-8800

- Since the expansion screws of the screw coupling SPE 2250-8800 are Geometry coated

- They must not be secured with Loctite and must be tightened using the torque according to the DOC drawing. see TPI_2014007

### Caution

- The corresponding tightening torque and the necessary screw retention must be entered in all new drawings and parts lists.

- In the case of changes, the correct values must be amended or corrected.

- This information must be adapted in the operating instructions, too, and be distributed in the assembly and customer service departments and to all service technicians.